Hardware Voice Management for DirectSound3D

DirectX 5 first provided the opportunity for hardware manufacturers to accelerate the DirectSound3D API (DS3D) through DirectSound’s Hardware Abstraction Layer (HAL).  However, based on feedback from application programmers, it is clear that supporting 3D audio hardware using DirectSound3D can be difficult with the existing API, primarily due to the voice/buffer management problem. The problem can be summarized as follows:  When an application uses DS3D on a system that has hardware acceleration, DirectSound will only allow the application to open as many 3D audio buffers as there are 3D hardware voices on the sound card. The solution is to specify and provide advanced voice management capabilities for hardware acceleration and provide access to the voice management functions at an API level.

In order to provide a timely and effective solution to this problem, Microsoft has provided the ability for  IHV’s to incorporate voice management for their existing and future products without requiring a new version of DirectSound.  The new voice manager, based on the existing DirectX architecture, will be accessible to application developers through a high-level API interface. This will allow:

- ISV’s  to easily take advantage of advanced voice management for DS3D in any application that ships with DirectX 5 or higher

- IHV’s to implement voice management at the driver level with limited changes in their existing device driver code base.

The voice management is achieved by the adoption of standardized Property Sets by IHV’s and the creation of an easy method for ISV’s to use these Property Sets.  An essential component of DirectSound in DirectX 5 was the inclusion of the Property Set Interface. (PSI)  This mechanism was designed to allow enhancements in functionality that are not directly supported by the DirectSound API to be gracefully exposed to an application.  By using the PSI, a device driver can specify data and features (“Properties”) that it wants to expose to the application.  The application queries the device to see if a particular PSI is supported by the device.  If it is, the application can then use the additional features provided by the device via the PSI.

The adoption of industry standard PSI’s for voice management and the definition of  the semantics of those Property Sets is critically important to the success of the PSI concept.  This allows application developers to check for only a single Property Set for the feature.  They can then utilize the voice manager with a single code-base, irrespective of particular hardware present in the system.  Without standardization, an application would have to query card after card  to search for voice management, and write special code to support each different piece of audio hardware depending on how the particular card supported voice management.

Equally important is to provide a mechanism in the form of a set of high-level API calls that allow developers access to these additional features easily and efficiently.  The property set interface is a low-level driver interface, and not particularly “programmer friendly.”  To provide high-level access to the voice manager, the property set interface is “wrapped” within a high-level API call, hiding the property set control in favor of a more familiar interface.  The wrapper code will be made freely available in source code format to ISV’s for integration into their applications.

To achieve the goals of industry standardization and developer acceptance, Microsoft is providing the following specifications and components:

- A standardized Property Set Interface definition for 3D Audio Hardware voice management consisting of:

- A GUID that uniquely defines the MS Voice management PSI

- A list of properties supported by the Property Set

- A specification of the voice management Property Set Interfaces

- Example code for an API-type Wrapper functions

The addition of standardized voice management significantly helps DirectSound3D become more widely supported by application developers.  By implementing it via property sets, the added functionality can be achieved within a relatively short time frame, utilizing the existing DirectX-5 and DirectX-6 platforms while maintaining forward compatibility with future DirectX versions.

Information on the Microsoft DirectSound3D buffer manager can be obtained by contacting directx@microsoft.com.

## Appendix A

Property Set Definitions

# The voice manager manages the allocation of audio buffers with hardware voices.  As buffers are created, they are not assigned to hardware voices until the Play method is invoked.    When in an advanced voice manager mode, the driver should report, via GetCaps,  the number of Free3DHWBUFFERS as a constant value equal to the total number of actual hardware buffers available on the system.

NOTE:  The behavior of the Create method, when invoked with either the DBSCAPS_LOCSOFTWARE or DBSCAPS_LOCHARDWARE remains unchanged, regardless of the state of the voice manager.  In other words,  only buffers created with neither the LOCHARDWARE nor LOCSOFTWARE flags set will be affected by the voice manager.

NOTE: The behavior of GetCaps should remain essentially unchanged, regardless of the mode of the voice manager.  I.e. the value reported for Free Hardware buffers should reflect the current state of the hardware.

# DSPROPSETID_VoiceManager

The properties that are defined for the voice manager property set interface are:

enum {

DSPROPERTY_VMANAGER_MODE,			// sets the mode for the manager

DSPROPERTY_VMANAGER_PRIORITY			// sets the priority for a buffer

DSPROPERTY_VMANAGER_STATE			// query current state of buffer

}

### DSPROPERTY_VMANAGER_MODE

The mode of the voice manager is determined by the DSPROPERTY_VMANAGER_MODE property.  It allows the application to select the behavior of the buffer manager depend on particular needs.  Since this property affects the operation of the buffer manager, setting this property on any buffer will affect the effect on all buffers; i.e. it is a global control.  The following modes are supported:

enum {

DSPROPERTY_VMANAGER_MODE_DEFAULT,		// set to default mode

DSPROPERTY_VMANAGER_MODE_AUTO,		// set to automatic priority mode

DSPROPERTY_VMANAGER_MODE_REPORT,		// set to report full mode

DSPROPERTY_VMANAGER_MODE_USER		// set to user settable priority mode

}

# DSPROPERTY_VMANAGER_MODE_DEFAULT

This mode turns off the voice manager and functionality reverts to DirectSound’s default behavior.  Only as many hardware buffers as there are hardware resources will opened.

# DSPROPERTY_VMANAGER_MODE_AUTO

This mode sets the voice manager to automatic mode.  In this mode, all calls to CreateSoundBuffer will succeed.  When a Play is called, if there are no free hardware voices, the buffer with the least time remaining to be played will be stopped.  The new sound will then be started on the newly freed hardware voice.  Since a looping sound by definition has an undetermined length, a looping sound will not be stopped.  If all hardware resources are occupied by looping sounds, the sound with the least time left to play will be stopped..   (NOTE TO DEVELOPERS:  If your application requires many looping sounds, it is recommended that you perform your own buffer management, or use the REPORT or USER modes).

# DSPROPERTY_VMANAGER_MODE_REPORT

This mode sets the voice manager to report mode.  In this mode, all calls to CreateSoundBuffer will succeed.  When Play is called,  the sound will be allocated to a free hardware resource and played.  If there are no free hardware voices, the Play call will fail.  The application can then determine on its own whether or not to stop a currently playing buffer.

# DSPROPERTY_VMANAGER_MODE_USER

This mode sets the voice manager to user priority mode.  This mode is designed to let the application programmer select which sounds will occupy the hardware resources in the event all resources are occupied based on a priority for each sound.  In this mode, all calls to CreateSoundBuffer will succeed. When Play is called,  the sound will be allocated to a free hardware resource and played. If  there are no free hardware voices, the driver must determine which currently playing buffer, if any, should be stopped and given to the new play request.  The determination is done based on the DSPROPERTY_VMANAGER_PRIORITY property value of the buffer and its relation to the currently playing buffers.  The default priority for a sound is priority 0.

- Priorities are ranked with 0 as the highest priority.

- A new sound can stop and replace an existing sound of the same priority.

- If more than one sound is determined to be the lowest priority, the sound with the least time left to play will be stopped

If (all resources busy) {

LowestPrioritySound = FindLowestPrioritySoundLeastTimeLeft();

If (NewSound->Priority <= LowestPrioritySound->Priority) {

Stop(LowestPrioritySound);

Start(NewSound);

Return(Success);

}

}

#### DSPROPERTY_VMANAGER_PRIORITY

The DSPROPERTY_VMANAGER_PRIORITY is used to set the priority of a sound.  Priorities are ranked with 0 (zero) as the highest possible priority.  A new sound of a higher priority will prematurely stop playback of a lower priority sound if all hardware resources are occupied when the new Play call is issued.

### DSPROPERTY_VMANAGER_STATE

The DSPROPERTY_VMANAGER_STATE property can be used to determine the state of the buffer.  The a buffer can be in one of the following states:

PLAYING3DHW:

The  buffer is currently sounding on a 3d hardware voice

SILENT:

The buffer is not currently sounding

BUMPED:

The buffer was playing, but is not currently sounding, because it has been stopped by a newer or higher priority sound.

PLAYFAILED

The buffer had a play attempted, but the play failed and the buffer never started playing.

enum {

DSPROPERTY_VMANAGER_STATE_PLAYING3DHW,

DSPROPERTY_VMANAGER_STATE_SILENT,

DSPROPERTY_VMANAGER_STATE_BUMPED,

DSPROPERTY_VMANAGER_STATE_PLAYFAILED

}

## Appendix B

Wrapper Code Functionality

The “wrapper” code allows a developer to easily access and control the property sets that define the voice manager and serves as example of how to use the property set interfaces.

### PSW_DSVManSetMode

The PSW_VManSetMode wrapper sets the mode for the property-set based voice manager.

HRESULT PSW_DSVManSetMode

(IDirectSound*	pDs,

DWORD 	Mode

DWORD*	pPrevMode);

Parameters

pDs

Pointer to the DirectSound object.

Mode

The mode of the voice manager.  Legal values are:

DSPROPERTY_VMANAGER_MODE_DEFAULT,		// set to default mode

DSPROPERTY_VMANAGER_MODE_AUTO,		// set to automatic priority mode

DSPROPERTY_VMANAGER_MODE_REPORT,		// set to report mode

DSPROPERTY_VMANAGER_MODE_USER		// set to user settable priority mode

pPrevMode

Address of variable which, upon return, will contain the previous mode of the voice manager, prior to the call being made.  An application that uses the voice manager should re-set the mode of the voice manager upon termination of the application.

Return Values

If PSW_VManSetMode is called on a DirectSound object that does not support the voice manager property sets or an invalid DirectSound object, a DirectSound error will be returned. Returns a COM error code.  The following error codes are not necessarily comprehensive.

DS_OK: The operation completed successfully.

Remarks

PSW_VManSetManMode must be able to open a secondary 3D buffer to properly set the voice manager.  If  it is unable to open the required buffer, it will fail.

### PSW_DSVManGetMode

The PSW_VManSetMode wrapper sets the mode for the property-set based voice manager.

HRESULT PSW_DSVManGetMode

(IDirectSound*	pDs,

DWORD 	*Mode);

Parameters

pDs

Pointer to the DirectSound object.

pMode

Address of variable which, upon return, will contain the current mode of the voice manager, prior to the call being made.

Return Values

If PSW_DSVManGetMode is called on a DirectSound object that does not support the voice manager property sets or an invalid DirectSound object, a DirectSound error will be returned. Returns a COM error code.  The following error codes are not necessarily comprehensive.

DS_OK: The operation completed successfully.

Remarks

PSW_DSVManGetManMode must be able to open a secondary 3D buffer to properly get the voice manager.  If  it is unable to open the required buffer, it will fail.

### PSW_DSVManSetPriority

The PSW_DSVManSetPriority wrapper sets the priority of a specific DirectSound3D buffer.

HRESULT DSPSW_VManSetPriority

(IDirectSoundBuffer*	pBuffer,

DWORD 	Priority

DWORD*	pPrevPriority);

Parameters

pBuffer

Pointer to a DirectSound3D object.

Priority

The relative priority for this sound buffer:  0 (zero) is the highest possible priority.

pPrevPriority

Pointer to a value to receive the previous priority for this buffer.

Return Values

If PSW_DSVManSetPriority is called on a DirectSound object that does not support the voice manager property set, a DirectSound error will be returned. Returns a COM error code.  The following error codes are not necessarily comprehensive.

DS_OK: The operation completed successfully.

Remarks

### PSW_DSVManGetPriority

The PSW_DSVManSetPriority wrapper sets the priority of a specific DirectSound3D buffer.

HRESULT DSPSW_VManSetPriority

(IDirectSoundBuffer*	pBuffer,

DWORD* 	pPriority);

Parameters

pBuffer

Pointer to a DirectSound3D object.

pPriority

Pointer to a value to receive the priority for this buffer.

Return Values

If PSW_DSVManGetPriority is called on a DirectSound object that does not support the voice manager property set, a DirectSound error will be returned. Returns a COM error code.  The following error codes are not necessarily comprehensive.

DS_OK: The operation completed successfully.

Remarks

### PSW_DSVManGetState

The PSW_DSVManGetState returns the current state of a specific DirectSound3D buffer.

HRESULT PSW_DSVManGetState

(IDirectSound*	pBuffer,

DWORD*	pState);

Parameters

pBuffer

Pointer to a DirectSound3D object.

pState

Pointer to a DWORD that will contain the state of the buffer.

Return Values

If PSW_VManGetState is called on a DirectSound object that does not support the voice manager property set, a DirectSound error will be returned. Returns a COM error code.  The following error codes are not necessarily comprehensive.

DS_OK: The operation completed successfully.

Remarks

Upon successful completion, *pState will contain the current state of the buffer. Defined states are as follows:

PLAYING3DHW:

The  buffer is currently sounding on a 3d hardware voice

SILENT:

The buffer is not currently sounding

BUMPED:

The buffer was playing, but is not currently sounding, because it has been stopped by a newer or higher priority sound.

PLAYFAILED

The buffer had a play attempted, but the play failed and the buffer never started playing.

Example Initialization Code

if (FAILED(DirectSoundCreate(NULL,&lpDS,NULL)))

{

DSoundErrorReport(hRes);

return 0;

}

if (!lpDS)

return 0;

if (FAILED(  PSW_DSVManSetMode(lpDS,

DSPROPERTY_VMANAGER_MODE_AUTO, &dPrevMode)))

{

// voice manager not supported on this card

} else

{

// Voice manager is now in Automatic mode.   Use DS3D calls from this point. 3D Sounds will //play only in hardware.

}

## Appendix C

Wrapper Code Example Source Code

NOTE: The following code is provided as example code only for the convenience of application developers.

/***************************************************************************

*

*  GetPropertySetInterface

*

*  Description:

*      Creates a property set interface from a DirectSound object.

*

*  Arguments:

*      LPDIRECTSOUND [in]: Pointer to the DirectSound object.

*      LPKSPROPERTYSET * [out]: Receives pointer to the property set

*                               interface.  The caller is responsbile for

*                               releasing this interface.

*

*  Returns:

*      HRESULT: DirectSound/COM result code.

*

***************************************************************************/

HRESULT GetPropertySetInterface(LPDIRECTSOUND lpDirectSound, LPKSPROPERTYSET *lplpKsPropertySet)

{

const WORD              awChannels[]        = { 1, 2 };

const UINT              cChannels           = sizeof(awChannels) / sizeof(awChannels[0]);

const DWORD             adwSamplesPerSec[]  = { 8000, 11025, 22050, 44100 };

const UINT              cSamplesPerSec      = sizeof(adwSamplesPerSec) / sizeof(adwSamplesPerSec[0]);

const WORD              awBitsPerSample[]   = { 8, 16 };

const UINT              cBitsPerSample      = sizeof(awBitsPerSample) / sizeof(awBitsPerSample[0]);

LPDIRECTSOUNDBUFFER     lpBuffer            = NULL;

DSBUFFERDESC            dsbd;

WAVEFORMATEX            wfx;

UINT                    a, b, c;

HRESULT                 hr;

// Create the 3D buffer.  DirectSound doesn't currently support querying

// for an IKsPropertySet interface from the DirectSound object, so we

// need to create a buffer first.

dsbd.dwSize = sizeof(dsbd);

dsbd.dwFlags = DSBCAPS_LOCHARDWARE | DSBCAPS_CTRL3D;

dsbd.dwBufferBytes = 4;

dsbd.dwReserved = 0;

dsbd.lpwfxFormat = &wfx;

// We'll loop through all "standard" formats until one of them works

wfx.wFormatTag = WAVE_FORMAT_PCM;

wfx.cbSize = 0;

for(a = 0; a < cChannels; a++)

{

wfx.nChannels = awChannels[a];

for(b = 0; b < cSamplesPerSec; b++)

{

wfx.nSamplesPerSec = adwSamplesPerSec[b];

for(c = 0; c < cBitsPerSample; c++)

{

wfx.wBitsPerSample = awBitsPerSample[c];

wfx.nBlockAlign = wfx.nChannels * wfx.wBitsPerSample / 8;

wfx.nAvgBytesPerSec = wfx.nSamplesPerSec * wfx.nBlockAlign;

hr = IDirectSound_CreateSoundBuffer(lpDirectSound, &dsbd, &lpBuffer, NULL);

if(SUCCEEDED(hr))

{

break;

}

}

if(SUCCEEDED(hr))

{

break;

}

}

if(SUCCEEDED(hr))

{

break;

}

}

// At this point, we have a 3D hardware buffer.  Let's try to get an

// IKsPropertySet interface from it.

if(SUCCEEDED(hr))

{

hr = IDirectSoundBuffer_QueryInterface(lpBuffer, &IID_IKsPropertySet, (LPUNKNOWN *)lplpKsPropertySet);

}

// Clean up

if(lpBuffer)

{

IDirectSoundBuffer_Release(lpBuffer);

}

return hr;

}

/***************************************************************************

*

*  PSW_DSVManGetMode

*

*  Description:

*      Sets the mode for the property-set based voice manager.

*

*  Arguments:

*      LPDIRECTSOUND [in]: Pointer to the DirectSound object.

*      VmMode * [out]: Address of variable which, upon return, will contain

*                      the current mode of the voice manager, prior to the

*                      call being made.

*

*  Returns:

*      HRESULT: DirectSound/COM result code.

*

***************************************************************************/

HRESULT PSW_DSVManGetMode(LPDIRECTSOUND lpDirectSound, VmMode *lpMode)

{

LPKSPROPERTYSET         lpKsPropertySet = NULL;

HRESULT                 hr;

// Get the property set interface

hr = GetPropertySetInterface(lpDirectSound, &lpKsPropertySet);

// Get the current mode

if(SUCCEEDED(hr))

{

hr = IKsPropertySet_Get(lpKsPropertySet, &DSPROPSETID_VoiceManager, DSPROPERTY_VMANAGER_MODE, NULL, 0, lpMode, sizeof(*lpMode), NULL);

}

// Clean up

if(lpKsPropertySet)

{

IKsPropertySet_Release(lpKsPropertySet);

}

return hr;

}

/***************************************************************************

*

*  PSW_DSVManSetMode

*

*  Description:

*      Sets the mode for the property-set based voice manager.

*

*  Arguments:

*      LPDIRECTSOUND [in]: Pointer to the DirectSound object.

*      VmMode [in]:  The mode of the voice manager.

*      VmMode * [out]: Address of variable which, upon return, will contain

*                      the previous mode of the voice manager, prior to the

*                      call being made.  An application that uses the voice

*                      manager should re-set the mode of the voice manager

*                      upon termination of the application.

*

*  Returns:

*      HRESULT: DirectSound/COM result code.

*

***************************************************************************/

HRESULT PSW_DSVManSetMode(LPDIRECTSOUND lpDirectSound, VmMode Mode, VmMode *lpPrevMode)

{

LPKSPROPERTYSET         lpKsPropertySet = NULL;

HRESULT                 hr;

// Get the property set interface

hr = GetPropertySetInterface(lpDirectSound, &lpKsPropertySet);

// Get the current mode

if(SUCCEEDED(hr) && lpPrevMode)

{

hr = IKsPropertySet_Get(lpKsPropertySet, &DSPROPSETID_VoiceManager, DSPROPERTY_VMANAGER_MODE, NULL, 0, lpPrevMode, sizeof(*lpPrevMode), NULL);

}

// Set the new mode

if(SUCCEEDED(hr))

{

hr = IKsPropertySet_Set(lpKsPropertySet, &DSPROPSETID_VoiceManager, DSPROPERTY_VMANAGER_MODE, NULL, 0, &Mode, sizeof(Mode));

}

// Clean up

if(lpKsPropertySet)

{

IKsPropertySet_Release(lpKsPropertySet);

}

return hr;

}

/***************************************************************************

*

*  PSW_DSVManGetPriority

*

*  Description:

*      Gets the priority of a specific DirectSound3D buffer

*

*  Arguments:

*      LPDIRECTSOUNDBUFFER [in]: Pointer to the DirectSoundBuffer object.

*      LPDWORD [out]: Pointer to a value to receive the priority for this

*                     buffer.

*

*  Returns:

*      HRESULT: DirectSound/COM result code.

*

***************************************************************************/

HRESULT PSW_DSVManGetPriority(LPDIRECTSOUNDBUFFER lpBuffer, LPDWORD lpdwPriority)

{

LPKSPROPERTYSET         lpKsPropertySet = NULL;

HRESULT                 hr;

// Get the property set interface

hr = IDirectSoundBuffer_QueryInterface(lpBuffer, &IID_IKsPropertySet, (LPUNKNOWN *)&lpKsPropertySet);

// Get the current priority

if(SUCCEEDED(hr))

{

hr = IKsPropertySet_Get(lpKsPropertySet, &DSPROPSETID_VoiceManager, DSPROPERTY_VMANAGER_PRIORITY, NULL, 0, lpdwPriority, sizeof(*lpdwPriority), NULL);

}

// Clean up

if(lpKsPropertySet)

{

IKsPropertySet_Release(lpKsPropertySet);

}

return hr;

}

/***************************************************************************

*

*  PSW_DSVManSetPriority

*

*  Description:

*      Sets the priority of a specific DirectSound3D buffer

*

*  Arguments:

*      LPDIRECTSOUNDBUFFER [in]: Pointer to the DirectSoundBuffer object.

*      DWORD [in]: New priority for this buffer.

*      LPDWORD [out]: Pointer to a value to receive the previous priority

*                     for this buffer

*

*  Returns:

*      HRESULT: DirectSound/COM result code.

*

***************************************************************************/

HRESULT PSW_DSVManSetPriority(LPDIRECTSOUNDBUFFER lpBuffer, DWORD dwPriority, LPDWORD lpdwPrevPriority)

{

LPKSPROPERTYSET         lpKsPropertySet = NULL;

HRESULT                 hr;

// Get the property set interface

hr = IDirectSoundBuffer_QueryInterface(lpBuffer, &IID_IKsPropertySet, (LPUNKNOWN *)&lpKsPropertySet);

// Get the current priority

if(SUCCEEDED(hr) && lpdwPrevPriority)

{

hr = IKsPropertySet_Get(lpKsPropertySet, &DSPROPSETID_VoiceManager, DSPROPERTY_VMANAGER_PRIORITY, NULL, 0, lpdwPrevPriority, sizeof(*lpdwPrevPriority), NULL);

}

// Set the new priority

if(SUCCEEDED(hr))

{

hr = IKsPropertySet_Set(lpKsPropertySet, &DSPROPSETID_VoiceManager, DSPROPERTY_VMANAGER_PRIORITY, NULL, 0, &dwPriority, sizeof(dwPriority));

}

// Clean up

if(lpKsPropertySet)

{

IKsPropertySet_Release(lpKsPropertySet);

}

return hr;

}

/***************************************************************************

*

*  PSW_DSVManGetState

*

*  Description:

*      Gets the state of a specific DirectSound3D buffer

*

*  Arguments:

*      LPDIRECTSOUNDBUFFER [in]: Pointer to the DirectSoundBuffer object.

*      VmState * [out]: Pointer to a value to receive the state for this

*                       buffer.

*

*  Returns:

*      HRESULT: DirectSound/COM result code.

*

***************************************************************************/

HRESULT PSW_DSVManGetState(LPDIRECTSOUNDBUFFER lpBuffer, VmState *lpState)

{

LPKSPROPERTYSET         lpKsPropertySet = NULL;

HRESULT                 hr;

// Get the property set interface

hr = IDirectSoundBuffer_QueryInterface(lpBuffer, &IID_IKsPropertySet, (LPUNKNOWN *)&lpKsPropertySet);

// Get the current priority

if(SUCCEEDED(hr))

{

hr = IKsPropertySet_Get(lpKsPropertySet, &DSPROPSETID_VoiceManager, DSPROPERTY_VMANAGER_STATE, NULL, 0, lpState, sizeof(*lpState), NULL);

}

// Clean up

if(lpKsPropertySet)

{

IKsPropertySet_Release(lpKsPropertySet);

}

return hr;

}